### JEREMIAH NYAGAH NATIONAL POLYTECHNIC

### ELECTRICAL AND ELECTRONICS DEPARTMENT

AUTOMATE AN ELECTRICAL MACHINE

PRACTICAL ASSESSMENT

### 50 MARKS

Qualification Code: 071306T4EEN

Qualification: Electrical Engineering Level 6
Unit Code: ENG/OS/PO/CR/05/6

Unit of Competency: Automate an Electrical Machine

INSTRUCTIONS TO THE CANDIDATE

- Time allocated: 3 hours.

- Use provided plain paper for drawings.

- Assessor will record performance via audio-visual means.

- Comply with I.E.E regulations.

Resources Provided:

- PLC (1)

- Start/Stop buttons (NO/NC)

- Magnetic valves (MV1, MV2)

- Agitator motor (M)

- Level transmitters (LT1, LT2, LT3)

- 4.0 mm² & 1.5 mm² PVC cables (Red, Black, Yellow/Green)

- Mini-trunking, screws, multimeter

- Installation board (1m x 1m)

### Problem: Mixing Process Automation

System Description:
Automate a tank mixing process (Figure 1) where:

- Start button opens MV1 (water inlet).

- At LT2: Close MV1, start agitator (M) and conveyor motor.

- At LT3: Stop conveyor, open MV2 (outlet).

- Below LT2: Stop agitator. Cycle repeats when Start is pressed again.

Figure 1

### TASKS

#### TASK 1: Design (7 Marks)

- Draw:

  - Block diagram of the system.

  - Wiring diagram (PLC, valves, motors, sensors).

#### TASK 2: PLC Programming (15 Marks)

- Write a ladder logic program to:

  - Use Start to initiate the cycle.

  - Control valves/motors based on sensor inputs (LT1–LT3).

  - Ensure agitator stops only when level < LT2.

- Label all operands

#### TASK 3: Simulation	(10 Marks)

- Simulate using software to verify logic.

#### TASK 4: Hardware Installation (10 Marks)

- Mount components on the board per your design.

- Secure wiring in trunking.

#### TASK 5: Testing	(8 Marks)

- Download the program to the PLC.

- Test:

  - MV1 opens/closes with Start/LT2.

  - Agitator starts at LT2, stops below LT2.

  - MV2 opens at LT3.