### JEREMIAH NYAGAH NATIONAL POLYTECHNIC

### ELECTRICAL AND ELECTRONICS DEPARTMENT

AUTOMATE AN ELECTRICAL MACHINE

PRACTICAL ASSESSMENT I

### 50 MARKS

Qualification Code: 071306T4EEN

Qualification: Electrical Engineering Level 6
Unit Code: ENG/OS/PO/CR/05/6

Unit of Competency: Automate an Electrical Machine

INSTRUCTIONS TO THE CANDIDATE

- Time Allocated: 3 hours.

- Use provided materials only.

- Comply with I.E.E regulations.

- Assessor will record critical steps via audio-visual means.

Resources Provided:

- PLC (1)

- Conveyor motor (M1)

- Clamping solenoid (SOL1)

- Drill motor (M2)

- Linear actuator for drill movement (M3)

- Limit switches (LS1: workpiece position, LS2: drill fully raised, LS3: drill fully lowered)

- Thermal overload sensor (OL1)

- Red indicator light (L1)

- Start (NO) and Stop (NC) push buttons

- 4.0 mm² & 1.5 mm² PVC cables

- Mini-trunking, multimeter, tools

### Problem: Automated Drilling Machine

System Requirements:

- Cycle Start:

  - Press Start PB → Conveyor (M1) moves workpiece into position (detected by LS1).

  - Stop M1 → Clamp workpiece (SOL1 activates).

  - Start drill motor (M2) → Lower drill (M3 extends until LS3).

- Safety:

  - If thermal overload (OL1) occurs:

    - Immediately stop all motors.

    - Turn on red light (L1).

- Cycle Completion:

  - Retract drill (M3 retracts until LS2) → Stop drill motor (M2) → Unclamp (SOL1 deactivates).

  - Repeat for next workpiece until Stop PB is pressed.

Figure

### TASKS

#### TASK 1: Design (7 Marks)

- Draw:

  - Block diagram (PLC, sensors, actuators).

  - Wiring diagram (Include power supply, I/O connections, emergency light circuit).

#### TASK 2: PLC Programming (15 Marks)

- Write ladder logic to:

  - Implement the drilling cycle with Start/Stop control.

  - Integrate thermal overload protection (OL1 → L1 + halt).

  - Use limit switches (LS1-LS3) for position feedback.

- Label operands.

#### TASK 3: Simulation (10 Marks)

- Simulate using software (e.g., TIA Portal) to verify:

  - Drill movement sequence (extend/retract).

  - Overload response (L1 activation).

#### TASK 4: Hardware Installation (10 Marks)

- Mount components on the board.

- Wire neatly with color coding (e.g., red for power, black for grounds).

#### TASK 5: Testing (8 Marks)

- Download program to PLC.

- Test:

  - Full drilling cycle (1 workpiece).

  - Overload simulation (trigger OL1 → check L1 and motor halt).