### JEREMIAH NYAGAH NATIONAL POLYTECHNIC

### ELECTRICAL AND ELECTRONICS DEPARTMENT

PRACTICAL ASSESSMENT II

### 50 MARKS

Qualification Code: 071306T4EEN

Qualification: Electrical Engineering Level 6
Unit Code: ENG/OS/PO/CR/05/6

Unit of Competency: Automate an Electrical Machine

INSTRUCTIONS TO THE CANDIDATE

- Time Allocated: 3 hours.

- Use provided materials only.

- Comply with I.E.E regulations.

- Assessor will record critical steps via audio-visual means.

Resources Provided:

- PLC (1)

- Conveyor motor (M1)

- Solenoid valve (for milk filling)

- Capping piston (M2)

- Labeling piston (M3)

- Bottle sensors (S1, S2, S3)

- Cap/Label sensors (2)

- Start/Stop push buttons (NO/NC)

- 4.0 mm² & 1.5 mm² PVC cables

- Mini-trunking, multimeter, tools

### Problem: Milk Bottle Filling & Capping Automation

System Requirements:

- Auto Mode:

  - Conveyor (M1) runs until S1 detects a bottle → Stop M1 → Open valve to fill milk → Stop valve after level → Restart M1.

  - At S2: Stop M1 → Activate capping piston (M2) until cap sensor detects → Restart M1.

  - At S3: Stop M1 → Activate labeling piston (M3) until label sensor detects → Restart M1.

- Stop PB: Halts the cycle after completing current bottle.

### TASKS

#### TASK 1: Design (7 Marks)

- Draw:

  - Block diagram (PLC, sensors, actuators).

  - Wiring diagram (Include power supply, I/O connections).

#### TASK 2: PLC Programming (15 Marks)

- Write a ladder logic program to:

  - Start conveyor with Start PB.

  - Implement auto-mode sequence (S1 → Fill → S2 → Cap → S3 → Label).

  - Stop conveyor immediately with Stop PB.

- Label all operands.

#### TASK 3: Simulation (10 Marks)

- Simulate using software to verify:

  - Sensor-triggered valve/piston actions.

  - Conveyor start/stop logic.

#### TASK 4: Hardware Installation (10 Marks)

- Mount components on the board.

- Wire according to your diagram (neatness, color coding).

#### TASK 5: Testing (8 Marks)

- Download program to PLC.

- Test:

  - Auto-mode sequence (1 full cycle).

  - Emergency stop functionality.