powerlines poe exams

POE 1,POE 2 and POE 3

April 22, 2025

DERRICK KARIUKI

## ✅ POE EXAM 1 – SIMPLE ANSWERS

### 1. Five elements of a transmission line

- Conductors

- Insulators

- Line supports

- Protective devices (like lightning arresters)

- Earth wire

### 2. Difference between short, medium, and long transmission lines

- Short: Less than 80 km

- Medium: 80–250 km

- Long: More than 250 km

### 3. What is a transmission line?

It’s a system that carries electric power over long distances.

### 4. Four types of line supports

- Wooden poles

- Steel towers

- RCC poles

- Latticed towers

### 5. Three materials for making line supports

- Wood

- Steel

- Concrete

### 6. Five types of transmission conductors

- ACSR

- Copper

- All-aluminum

- AAAC

- Steel-core aluminum

### 7. Factors affecting sag

- Temperature

- Wind pressure

- Weight of conductor

- Span length

- Ice loading

### 8. Four types of electrical joints

- Britannia joint

- Western Union joint

- Tee joint

- Married joint

### 9. Three types of electrical loads

- Resistive (e.g. heaters)

- Inductive (e.g. motors)

- Capacitive (e.g. capacitor banks)

### 10. What is transmission line termination?

It means connecting the end of the transmission line to a device or ground.

### 11. Three types of termination

- Indoor

- Outdoor

- Cable type termination

### 12. Importance of termination

To ensure safe and reliable connection.

### 13. AC vs DC transmission

- AC: Power changes direction; easy to step up/down

- DC: Flows in one direction; less loss over long distance

### 14. AC Transmission

- ✅ Advantages: Easy voltage control, transformers can step up/down

- ❌ Disadvantages: Has more losses, reactive power

### 15. DC Transmission

- ✅ Advantages: Less power loss, better for long distances

- ❌ Disadvantages: More expensive equipment, harder to change voltage

### 16. Testing in transmission lines

- Continuity test

- Insulation test

### 17. Four systems of power transmission

- AC Overhead

- AC Underground

- DC Overhead

- DC Underground

### 18. Four important components of overhead transmission line

- Conductor

- Insulators

- Line supports (like poles or towers)

- Earth wire

### 19. Four advantages of pin-type insulators

- Simple construction

- Low cost

- Easy to install

- Suitable for voltages up to 33kV

### 20. (Same as 19, just continued on a new line)

So, no need to repeat — already answered.

### 21. Two purposes of suspension insulators

- Used for high voltage lines (above 33kV)

- They provide flexibility and greater mechanical strength

### 22. Three methods of improving string efficiency

- Use of grading rings

- Use of longer cross-arms

- Capacitance grading (different-sized discs)

### 23. Two methods of reducing corona effect

- Use of bundled conductors

- Use of larger diameter conductors

### 24. (Same as 23 – repeated formatting)

So already covered above.

### 25. Define “sag”

The vertical distance between the straight line joining two supports and the lowest point of the conductor.

### 26. Two factors affecting sag

- Temperature (hot weather = more sag)

- Weight of conductor or loading (ice, wind)

### 27. Three classifications of overhead lines with voltage levels

- Low Voltage – below 1kV

- Medium Voltage – 1kV to 69kV

- High Voltage – 69kV to 230kV

- Extra High Voltage – above 230kV

(Any 3 from these is okay)

### 28. Four properties of conductor materials

- High electrical conductivity

- High mechanical strength

- Corrosion resistant

- Lightweight

### 29. Corona Loss Calculation

Let me know if you want me to solve this step-by-step — it needs formulas:

- Use Peek’s formula

- Given: radius, spacing, pressure, temperature, m₀ = 0.85

### 30. Sag Calculation with wind & ice

Also requires formulas. Given:

- Span = 275 m

- Weight = 0.865 kg/m

- Diameter = 1.96 cm

- Ice thickness = 1.27 cm

- Wind pressure = 3.9 g/cm²

- Factor of safety = 2

## Ice density = 0.91 g/cm³

## ✅ POE EXAM 2 – SIMPLE ANSWERS

### 🔹 SECTION A – Answer All (40 Marks)

- Systems of power transmission

- AC overhead

- AC underground

- DC overhead

- DC underground

- Components of overhead transmission line

- Conductor

- Insulators

- Towers/poles

- Earth/ground wire

- Advantages of pin-type insulator

- Simple design

- Cheap to install

- Easy maintenance

- Suitable for low voltages

- Purpose of suspension insulators

- Support high voltage conductors

- Provide flexibility and withstand mechanical load

- Methods of improving string efficiency

- Use of grading rings

- Using longer cross-arms

- Increasing number of insulator discs

- Methods of reducing corona effect

- Use thicker/larger diameter conductors

- Use bundled conductors

- What is sag? It’s the vertical distance between the highest point of support and the lowest point of the conductor.

- Factors affecting sag

- Temperature

- Weight of conductor

- Classifications of overhead lines

- Low voltage: < 1kV

- Medium voltage: 1–69kV

- High voltage: 69–230kV

- Extra high voltage: >230kV

- Properties of conductor materials

- High conductivity

- Strong/tensile strength

- Corrosion resistance

- Lightweight

- ✅ POE EXAM 3 – SIMPLE ANSWERS

### 🔹 SECTION A – Answer All (40 Marks)

- Considerations for High Voltage  lines

- Proper insulation

- Right conductor size

- Enough clearance

- Minimize losses

- Effects of vibration

- Loosens fittings

- Damages conductor

- Creates noise

- Causes fatigue/failure

- Elements of High Voltage line

- Conductors

- Insulators

- Towers/poles

- Ground wire

- Causes of voltage surges

- Lightning

- Switching

- Faults

- Load variations

- Types of power lines

- Short: Less than 80 km

- Long: More than 250 km

- MCBs vs fuses

- Reusable

- Faster trip

- Safer

- More accurate

- Define:

- Current rating: Max safe current

- Fusing factor: Fuse current ÷ normal current

- Tensions affecting conductors

- Wind

- Ice load

- Temperature

- Span length

- Requirements of switchgear

- Reliable

- Fast operation

- Safe

- Can interrupt large currents

- Ways to reduce corona

- Use bundled conductors

- Use smooth surface conductors

- Increase spacing

- Use larger conductors